Principal Changes in IS 398-1:2017

- Categories A, B, C & D have been redefined in terms of the extent of susceptibility of expansion of the hardcore, rather than the magnitude of expansion, and the potential of future damage to the building from pyritic heave.  The “red”, “amber”, “green” colour coding has been removed.

- Where possible, definitive conclusions are provided – Categories A, B & D.

- The potential future damage for buildings assigned to Category B is now deemed limited to minor damage, defined as easily treated using periodic repairs e.g. cracks easily filled, repointing of external masonry, etc.

- For buildings assigned to Category B it is not envisaged that replacement of the hardcore would be necessary.  For buildings assigned to Category C it is not possible to be so definitive.

- The classifications have been revised as follows:

| IS 398-1:2013 | IS 398-1:2017 |
| --- | --- |
| Pass – not susceptible to expansion | Negligible susceptibility to expansion (pass) |
| Inconclusive –susceptible to limited expansion | Low susceptibility to expansion |
|  | Medium susceptibility to expansion |
| Fail –susceptible to significant expansion | Significant susceptibility to expansion (fail) |

As a result the term “Inconclusive” is no longer applicable to the classification of the hardcore.

The Building Categorisation Matrix has been revised accordingly.  See Table 6 from IS 398-1:2017 attached.

- The size of the sampling hole through the ground floor slab has been increased to facilitate sampling to the full depth of the hardcore.

- There have been changes to the laboratory Test Suites and an option has been added to allow the Geologist to instruct additional testing.

- The criteria for the evaluation of visual damage assessment have been revised in Annex B.  As a result there is an increase in the number and type of attributes evaluated for damage during the assessment to provide more comprehensive and reliable criteria for assessing pyrite-induced damage in buildings.

- The Certificate of Building Categorisation in Annex F has been revised to include a signature of the Professional Geologist /Chartered Geotechnical Engineer and to take account of other relevant revisions made to the document.  See Annex F from IS 398-1:2017 attached.

- A new informative Annex I has been added to assist the Professional Geologist /Chartered Geotechnical Engineer for interpretation of the chemical and petrographic results and to assist with the classification of the hardcore sample.

- The Flowchart outlining the Process for assessing, testing and categorisation of buildings has been revised for ease of use.